HEADER DATA

BOREHOLE TYPE

Fully cored	FC

Open / chip	OC

Partly cored	PC

Reverse Circulation	RC

DATA STATUS

Raw / Uncorrected	R

Adjusted to geophysics	A

Seams adjusted to geophysics	S

Corrected to verticality	V

Final	F

Unknown	U

BOREHOLE PURPOSE

Blasthole	BH

Coal Quality	CQ

large diameter 	CL

slim core testing	CS

spontaneous combustion testing	CC

Environmental	EN

acid leachate testing	EA

stygofauna monitoring	ES

Gas	GS

compliance gas testing	GC

controlled pressure well	GL

end of borehole well	GE

gas drainage undifferentiated	GD

goaf drainage	GG

pumping inert gas into workings	GN

ranging well	GR

surface to in-seam well	GI

underground in-seam gas-riser	GU

vertical production well	GZ

virgin gas testing	GV

Geotech	TG

extensometer	TE

geotechnical properties	TR

penetrometer 	TP

primary hydraulic fracturing	TF

pumping pre-consolidation grout	TC

stress test cell / stress overcore	TX

tiltmeter	TL

Hydrological	HY

multi-channel vibrating wire	HM

piezometer

nested standpipe piezometer	HN

production water bore	HW

standpipe piezometer	HS

vibrating wire piezometer	HV

Lox	LX

Service	SV

ballast	SB

cement	SC

electricity	SE

nitrogen	SN

refuge	SR

plug	SP

stone dust	SD

Structure	ST

fault delineation	SF

intrusion delineation	SI

GEODETIC DATUM

Australian Geodetic Datum	AGD

Australian Geodetic Datum 66	AGD66

Australian Geodetic Datum 84	AGD84

Australian Mapping Grid 	AMG

Geocentric Datum Australia	GDA

Geocentric Datum Australia 94	GDA94

Geocentric Datum Australia 2020	GDA2020

Local Datum	LOC

Map Grid Australia	MGA

Universal Transverse Mercator	UTM

HEIGHT DATUM

Australian Height Datum	AHD

Approximate Level 	APX

Local Datum	LOC

LOCATION ACC

approximate	A

barometric	B

digitized	D

GPS (hand held)	G

surveyed	S

GEOPHYS LOG

Acoustic Scanner	A

Caliper	C

Cement Bond Log	B

Density	D

Dipmeter	I

Downhole Camera	M

Full Waveform Sonic	F

Gyroscopic Verticality	Y

Magnetic Susceptibility	K

Natural Gamma	G

Neutron	N

Resistivity	R

Spontaneous Potential	P

Sonic	S

Temperature	T

Verticality	V

X-Ray	X

BOREHOLE STATUS

backfilled	B

casing removed	X

cemented	N

completed	C

equipment in borehole	E

hazard in borehole	H

infrastructure	I

in progress	P

mined	M

open	O

piezometer	Z

plugged	G

rehabilitated	R

unknown	U

water bore	W

CASING DATA

CASING MATERIAL

fibreglass	FB

not recorded	NR

pvc	PV

stainless steel 	SS

steel	ST

uncased	UC

CASING TYPE

perforated	P

slotted	S

threaded	T

CASING NAME

HWT thread class	HWT

Ozcon casing 	OZCO

PN06 class UPVC	PN06

PN09 class UPVC	PN09

PN12 class UPVC	PN12

PN18 class UPVC	PN18

SFJ thread class	SFJ

CASING GROUT

bentonite	BE

cement slurry 	CS

concrete aggregate 	AG

cuttings  	CT

gypsum	GY

pressure grouted slurry	PG

soil	SO

two pack foam	FO

washed gravel	GV

CEMENTING DATA

CEMENT METHOD

from surface	FS

poly reel	PR

sacrifice poly	SP

through drill rods	TR

DRILLING DATA

BIT TYPE

Non-Coring Bits

auger	AG

blades / drag blade	BL

hammer	HA

mill claw	MC

poly crystalline diamond open	PO

rock roller / tricone	TR

surface / wing	SF

unknown	UN

Coring Bits

diamond core (wireline)	DW

poly crystalline diamond core	PC

(conventional)

poly crystalline diamond core	PW

(wireline)

tungsten carbide core	TC

(conventional)

DRILL FLUID

air	A

bentonic mud	M

foam	F

polymer	P

soluble oil	S

water	W

water injection	I

DRILL SIZE NAME

Wireline Barrel

NQ (48mm / 76mm)	NQ

HQ (64mm / 96mm)	HQ

PQ (85mm / 123mm)	PQ

NQ triple tube (45mm / 76mm)	NQ3

HQ triple tube (61mm / 96mm)	HQ3

PQ triple tube (83mm / 123mm)	PQ3

Conventional Core  Barrel

NMLC triple tube (52mm / 76mm)	NMLC

HMLC triple tube (64mm / 99mm)	HMLC

PMLC triple tube ( / )	PMLC

3” conventional (76mm / 111mm)	3C

4” conventional (102mm / 140mm)4C

6” conventional ( / )	6C

8” conventional (203mm / 260mm)8C

10” conventional ( / )	10C

12” conventional (305mm / )	12C

Other

Unknown	UNK

WATER OBSERVATION DATA

WATER TEST TYPE

305mm Board	3

610mm Board	6

914mm Board	9

bucket	B

dry	D

estimate	E

driller injected	I

observed damp	M

v-notch	V

observed wet	W

SAMPLE  PURPOSE

Coal Quality

raw ply (coal, roof, floor or parting)	QP

bulk sample	QB

channel sample (underground)	QU

Loxline

raw ply (coal, roof, floor or parting)	LP

Spontaneous Combustion

raw ply (coal, roof, floor or parting)	SP

bulk sample	SB

channel sample	SU

Geotechnical Sample

laboratory tested	GT

field tested	GF

Water Quality Sample

laboratory tested	WT

field tested	WF

Gas Sample

exploration (virgin)	ME

compliance (drained)	MD

Environmental Sample

soil	ES

overburden characterisation	EO

(compliance)

reactive ground	ER

Other

age dating	AD

palynology	PN petrology	PE

INTERVAL STATUS

raw / uncorrected	R

adjusted to geophysics	A

interpreted from geophysics	I

unknown	U

LITHO TYPE

Unconsolidated Sediments

Clay	CL

Mud	MD

Silt	SI

Sand	SA

Gravel	GV

Cobbles	OB

Boulders	BO

Alluvium	AL

Colluvium	CV

Diatomaceous Earth	DE

Fill / Spoil	FI

Fireclay	FC

Loam	LO

Soil	SO

Carbonaceous Sediments

Coal	CO

Lignite	LG

Brown Coal	BC

Peat	PE

Burnt Wood / Charcoal	BW

Weathered Coal	CW

Oil Shale	OS

Tar Sand	TS

Coaly Claystone	ZC

Coaly Mudstone	ZM

Coaly Shale	ZH

Coaly Siltstone	ZT

Coaly Sandstone	ZS

Carbonaceous Claystone	XC

Carbonaceous Mudstone	XM

Carbonaceous Shale	XH

Carbonaceous Siltstone	XT

Carbonaceous Sandstone	XS

Clastic Sedimentary Rocks

Sedimentary Rock, undifferentiated	SU

Claystone	CS

Pellet Claystone	PC

Mudstone	MS

Shale	SH

Siltstone	ST

Sandstone	SS

Conglomerate	CG

Conglomerate (>65% matrix)	M1

Conglomerate (35-65% matrix)	M2

Conglomerate (<35% matrix)	M3

Breccia	BR

Fault Breccia	FB

Diamictite	DI

Tillite	TI

Chemical Sedimentary Rocks

Calcrete	CC

Carbonate	CB

Chalk	CK

Chert	CH

Cone in Cone Carbonate	KK

Dolomite	DM

Ferricrete	FK

Fossil Wood	FW

Ironstone	IS

Kaolinite	KA

Laterite	LA

Limestone	LS

Limonite	LI

Silcrete	SC

Tonstein	TN

Igneous

Igneous Rock, undifferentiated	IG

Volcanic Rock, undifferentiated	VR

Intrusive Rock, undifferentiated	IN

Acid / Felsic Volcanic	AV

Intermediate Volcanic	IV

Basic / Mafic Volcanic	BV

Acid / Felsic Intrusive	AI

Intermediate Intrusive	II

Basic / Mafic Intrusive	BI

Andesite	AN

Basalt	BS

Dolerite	DO

Gabbro	GB

Granite	GR

Granodiorite	GD

Mozonite	MZ

Rhyolite	RH

Syenite	SY

Trachyte	TR

Tuff	TF

Tuffite	TT

Volcanic Breccia	VB

Metamorphic

Metamorphic Rock, undifferentiated	MM

Basement Undifferentiated	BU

Mylonite	MY

Quartzite	QT

Slate	SL

Phyllite	PH

Schist	SZ

Gneiss	GN

Minerals

Calcite	CA

Gypsum

Pyrite	PY

Quartz	QZ

Siderite	SD

Talc	TA

Other

Core Loss	KL

Lost Coal (from geophysics)	LC

Missing Record	MR

Non Coal	NC

No Recovery	NR

Not Logged	NL

Old Workings	OW

Void	VD

LITHO QUAL

Coal Brightness

bright (>90%)	BR

bright with dull bands (60-90%)	BB

interbanded dull and bright 	BD

(40-60%)

mainly dull with frequent bright	DB

bands (10-40%)

dull with minor bright bands (1-10%)	DM

dull (<1%)	DD

mid-lustrous to bright	M1

mid-lustrous	M2

mid-lustrous to dull	M3

Other Coal

undifferentiated	CU

heat affected	HA coked	KC

cindered	CI

fusainous	FU

anthracite	AN

cannel (torbanite, bog)	CT

dull conchoidal	DC

inferior	IF

sapropelic (incl. cannel, torbanite, 	SP

boghead)

stony	SY

Grain Size

Unconsolidated Sediments

clayey	CL

silty	SI

sandy	SA

gravelly	GV

Sandstone / Sand / Gravel

very fine grained	VV

very fine to fine grained	VF

very fine to medium grained	VM

very fine to coarse grained	VC

very fine to very coarse grained	VX

fine grained	FF

fine to medium grained	FM

fine to coarse grained	FC

fine to very coarse grained	FX

medium grained	MM

medium to coarse grained 	MC

medium to very coarse grained	MX

coarse grained	CC

coarse to very coarse grained 	CX

very coarse grained	XX

Conglomerate / Alluvium

granular	GG

granular to pebbly	GP

granular to cobbly	GO

granular to bouldery	GU

pebbly	PP

pebbly to cobbly	PO

pebbly to bouldery	PU

cobbly	OO

cobbly to bouldery	OU

bouldery	UU

Tuff / Tuffite / Breccia / Fault Breccia

clay sized	CS

mud sized	MS

silt  sized	TS

sand sized	SS

SHADE

light	L

light to medium	A

light to dark	C

medium	E

medium to dark	B

dark	D

banded	N

mottled	M

speckled	S

variegated	V

HUE / COLOUR

blackish / black	K

bluish / blue	L

brownish / brown	B

buff	F

creamy / cream	C

greenish / green	E

greyish / grey	G

multi-coloured	M

off white	X

orangey / orange	O

pinkish / pink	P

purplish / purple	U

reddish / red	R

whitish / white	W

yellowish / yellow	Y

ADJECTIVE

Quantity

abundant (30-60%)	AB

common (15-30%)	CM

decreasing in abundance	DA

dominant (>60%)	DO

highly	HI

in part	IP

increasing in abundance	IA

large	LR

minor (1-15%)	MN

partially	PR

rare (<1%)	RA

slightly	TY

strongly	TG

thick	TK

thin	TH

very	VE

Appearance

altered	AL

bright	BR

clear	LC

coarser (<10% of unit)	XC

conchoidal	CC

dull	DD

fault gouge	FT

finer (<10% of unit)	FF

hard	HR

heat affected	HA

interbanded 	IB

irregular	IR

lustrous	LU

moderately	MO

opaque	OP

resinous	RS

soft	SO

translucent	TL

Lithological

acidic 	AC

arenitic	AR

arkosic	AK

basaltic	BS

basic	BC

bentonitic	BE

calcareous	CA

carbonaceous	XX

carbonate	CB

chloritic	CR

clayey	CL

claystone	CS

coal stringers	CX

coaly	CO

conglomeratic	CG

detrital	DE

dolomitic	DM

feldspathic	FS

ferruginous	FE

fossiliferous	FO

fusainous	FU

glauconitic	GC

graphitic	GP

illitic	IL

intermediate	IM

intrusive	IN

iron stained	ID

kaolinitic	KA

lateritic	LA

limonitic	LI

lithic	LT

loamy	LO

manganiferous	MG

marly 	MR

metamorphosed	MM

micaceous	MI

muddy	MD

mudstone	MS

oxidised	OX

peaty	PE

phosphatic	PP

pyritic	PY

quartzose	QZ

sandstone	SS

sandy	SA

shaly	SH

shelly	HY

sideritic	SD

siliceous	SC

silicified	SF

siltstone	ST

silty	SI

smectitic	SM

sooty	SX

stony	SY

sub arenitic	AM

tillitic	TI

tonsteinous	TN

tuffaceous	TF

vitrainous	VI

volcanic	VO

Inclusions

bands	BN

blebs	BL

clasts	CT

cobbles	OO

concretions	CI

disseminated	DS

fragments	FR

grains	GN

granules	GG

gravelly	GV

laminae (2-20mm)	LM

layers	LY

lenses	LN

matrix	MX

nodules	ND

partings	PA

pebbles	PB

pellets	PT

penny bands (<2mm)	PN

phases	PH

pods	PO

stringers	SG

traces	TR

wisps	WP

Preposition

and	ET

as	AS

of	OF

on	ON

with	WI

Position

alternating	AT

near base of unit	BU

near middle of unit	MU

near top and base of unit	XU

near top of unit	TU

tends to	TT

throughout	TO

LITHO INTERREL

coarsening up to	CU

coarsely interbedded (> 200mm) with	CB

disseminated with	DS

fining up to	FU

grading into	GD

interbedded with	IB

intercalated with	IC

interlaminated (< 20mm) with	IL

intermixed with	IM

irregularly interbedded with	IR

thinly interbedded (60-200mm) with	TB

with bands of	BN

with boulders of	BO

with cement of	CM

with clasts of	CT

with cobbles of	OO

with fragments of	FR

with granules of	GG

with lenses of	LN

with matrix of	MX

with nodules of	ND

with pebbles of	PB

with pods of	PO

with wisps of	WP

very thinly interbedded (20-60mm) with	UB

WEATHERING

residual soil	R

extremely weathered	E

highly weathered	H

distinctly weathered	D

moderately weathered	M

slightly weathered	S

weathered	W

fresh	F

EST  STRENGTH

Unconsolidated Cohesive

very soft	C1

soft	C2

firm	C3

stiff	C4

very stiff	C5

hard	C6

Unconsolidated Cohesionless

very loose	S1

loose	S2

medium dense	S3

dense	S4

very dense	S5

Rock

extremely low strength rock	R1

very low strength rock	R2

low strength rock	R3

medium strength rock	R4

high strength rock	R5

very high strength rock	R6

extremely high strength rock	R7

BED  SPACING

massive / absent bedding	MA

very thickly bedded (>2m)	VB

thickly bedded (600-2000mm)	CB

medium bedded (200-600mm)	MB

thinly bedded (60-200mm)	TB

very thinly bedded (20-60mm)	UB

thickly laminated (6-20mm)	LM

thinly laminated (<6mm)	LL

irregular spaced bedding	IR

DEFECT  TYPE

Natural

bedding plane	BP

broken zone	BZ

clay band	CL

coal cleat	CE

contraction fracture	CF

cross bedding	XB

dyke	DY

fault	FT

foliation	FO

fracture (undifferentiated)	FR

joint	JN

shear zone	SH

sill	SI

softened zone (non-tectonic)	SO

vein	VN

Induced and Non-Intact

discing	DS

drilling induced break	DB

drilling induced broken zone	DZ

DEFECT INTACT

intact	I

DEFECT  SPACING

extremely wide (>2m)	EW

very wide (600-2000mm)	VW

wide (200-600mm)	WI

moderately wide (60-200mm)	MW

moderately narrow (20-60mm)	MN

narrow (6-20mm)	NA

very narrow (<6mm)	VN

CORE  STATE

solid core	S

fragmented core	F

broken core	B

very broken core	V

crushed core	C

overdrilled core	O

cuttings	K

MECH  STATE

Slaking

non slakng	NS

low slaking	LS

medium slaking	MS

high slaking	HS

Plasticity

non plastic	NP

low plasticity	LP

intermediate plasticity	IP

high plasticity	HP

Other

blocky	BK

brecciated	BR

brittle	BL

cleated	CE

disintegrates on wetting	DW

expanding clay	EX

fissile	FS

fissured	FI

flaggy	FG

flaky	FL

fractured	FR

fretted	FT

friable	FB

indurated	IN

micro faulted	MF

non-cleated	NC

powdery	PO

puggy	PU

sheared	SH

slabby	SL

slickensided	SK

sticky	ST

subfissile	SF

TEXTURE

amorphous	AM

amygdaloidal	AG

aphanitic	AP

chalky	CK

cherty	CH

clast supported	CS

concretionary	CI

crystalline	XL

earthy	EA

equigranular	EQ

fibrous	FB

flow banded	FL

glassy	GS

granular	GG

gritty	GT

matrix supported	MS

nodular	ND

oolitic	OO

pelletal	PT

pisolitic	PS

platey	PL

porphyritic	PR

schistose	SZ

soapy	SO

vesicular	VS

vitreous	VT

vuggy	VU

waxy	WX

BASAL CONTACT

adheres at base	A

basal contact open or readily parts 	B

basal contact deformed	D

erosional basal contact	E

faulted at basal contact	F

gradational basal contact	G

sharp and irregular basal contact	I

jointed at basal contact	J

sharp and oblique basal contact	O

sharp and planar basal contact	P

fractured at basal contact	R

sheared at basal contact	S

sharp and undulose basal contact	U

SED  FEATURE

Bedding

contorted bedding	CT

convoluted bedding	CV

current bedding	CB

diffuse bedding	DF

disturbed bedding	DB

flasar bedding	FL

graded bedding	GB

lenticular bedding	LB

planar bedding	PL

poorly developed bedding	PD

ripple bedding	RI

wavy bedding	WB

well-developed bedding	WD

Cross Bedding

high angle cross bedding (>30°)	HX

medium angle cross bedding	MX

(10°-30°)

low angle cross bedding (<10°)	LX

cross bedding	XB

fine cross bedding	FX

tabular cross bedding	TX

trough cross bedding	RX

Laminations

large scale cross laminations (>2m)	LL

medium scale cross laminations	ML

(200 – 2000mm)

small scale cross laminations	SL

(<200mm)

wavy laminations	WL

Shape

very angular grains	VG angular grains	AG

subangular grains	GG

well rounded grains	WG

rounded grains	RG

subrounded grains	BG

bladed grains	DG

prolate grains	LG

tabular grains	TG

very angular fragments	VF

angular fragments	AF

subangular fragments	GF

well rounded fragments	WF

rounded fragments	RF

subrounded fragments	BF

very angular pebbles	VP

angular pebbles	AP

subangular pebbles	GP

well rounded pebbles	WP

rounded pebbles	RP

subrounded pebbles	BP

Sorting

well sorted 	WS

moderately sorted	MS

poorly sorted 	PS

bimodal sorting	BS

polymodal sorting 	YS

coarsening upwards	CU

fining upwards	FU

Permeability / Porosity

impermeable (<0.1mD)	IR

low permeability (0.1-10mD)	LP

medium permeability	MP

(10-10000mD)

high permeability (>10000mD)	HP

permeable 	PE

porous 	PO

Cracks

dessication cracks	DC

intraformational cracks	IC

mud casts / cracks	MC

shrinkage cracks	SC

syneresis cracks 	YC

Structures

bioturbated	BT

boudinage	BD

bounce marks / prod casts	PC

burrowing	BW

climbing ripples	CR

colloidal iron deposit	CI

compaction feature	CF

drop pebbles	DP

flame structures	FS

imbricate clasts	IM

load cast	LC

pebble lag 	PG

reworked	RW

ripple marks	RM

rip-up clasts 	RU

rootlet beds	RB

scour and fill	SF

sedimentary dyke	DY

slumping	SP

soft sediment deformation 	DE

stylolites 	ST

varving	VV

water escape structures	WE

Position

in part	IP

near base of unit	BU

near middle of unit	MU

near top and base of unit	XU

near top of unit	TU

throughout	TO

MIN FOS ABUND

dominant (>60%)	N

abundant (30-60%)	A

common (15-30%)	C

minor (1-15%)	M

minor (1-15%)	M

accessory 	E

secondary 	D

MIN FOS TYPE

Minerals

ankerite	AN

apatite	AP

bauxite	BA

biotite	BT

calcite	CA

carbonate	CB

chalcedony	CD

chalcopyrite	CC

chert	CH

chlorite	CR

clay	CL

common opal 	OP

dickite	DI

dolomite	DM

epidote 	EP

feldspar	FS

galena	GA

garnet 	GR

glauconite	GC

goethite 	GO

graphite	GP

gypsum	GY

haematite	HE

heavy minerals	HM

illite	IL

ilmenite	IM

iron oxide	IO

ironstone	IS

kaolinite	KA

limonite	LI

magnetite	MT

manganese	MG

marcasite	MC

mica	MI

montmorillonite	ML

muscovite	MV

olivine	OL

opaque minerals 	OM

orthoclase	OR

phosphates	PP

plagioclase	PG

pyrite	PY

quartz	QZ

siderite	SD

silica	SC

sulphides	SU

talc	TA

unidentified mineral	UN

vertabrata	VB

vivianite	VV

zeolite	ZE

Fossils

bivalves	BI

brachiopods	BR

bryozoans	BZ

carbonaceous remains	XR

carbonaceous root traces	RC

charcoal	FB

coprolites	CP

faecal remains 	FR

foraminifera	FM

fossil wood	FW

fossils	FO

gastropods	GT

marine fossils	MF

pelycepods	PE

plant fragments	PF

plant impressions	PI

resin	RS

resin aggregates	RA

root traces	RT

rootlets	RO

sediment filled root traces	SR

shells	HY

woody fragments 	WF

MIN FOS ASSOC

amorphous	AM

in amygdules	AG

bands	BN

cement	CM

clasts	CT

coarse grains	CC

coating	OU

concentrated at base	CB

concentrated at top 	CN

concretions	CI

cone in cone structure	KK

crystals	XL

detrital	DE

disseminated	DS

fibrous	FB

fine grains	FF

fragments	FR

glendonites	GD

grains	GN

in blebs	BL

in cavities	CV

in cleat	CE

in pods	PO

in veins	VN

in vesicles	VS

in vughs	VU

infilling fault discontinuities	FD

infilling of burrows 	IB

infilling vesicles	IV

intercalations	IC

laminae	LM

lenses	LN

matrix	MX

microflakes	MF

nodules	ND

on bedding planes	BP

on fracture planes	FP

on joints	JN

oolites 	OO

pebbles	PB

pellets	PT

phenocrysts 	PH

radial filaments 	FL

replacement 	RE

replacing fossils	RF

resinous	RS

rhombs	RH

staining	SN

throughout	TO

traces	TR

wisps 	WP

GAS

trace (<1m3/t)	T

low gas present (1-5m3/t)	L

moderate gas present (5-10m3/t)	M

high gas present (10-15m3/t)	H

very high gas present (>15m3/t)	V

H2S not detected 	N

H2S present 	P

RMU DATA

RMU TYPE

broken zone	B

core loss	L

core with defects	D

not recorded	N

open	O

soil properties	S

unbroken core	U

WEATHERING

residual soil	R

extremely weathered	E

highly weathered	H

distinctly weathered	D

moderately weathered	M

slightly weathered	S

weathered	W

fresh	F

ALTERATION

extremely altered	E

distinctly altered	D

slightly altered	S

altered	A

fresh	F

EST STRENGTH

Unconsolidated Cohesive

very soft	C1

soft	C2

firm	C3

stiff	C4

very stiff	C5

hard	C6

Unconsolidated Cohesionless

very loose	S1

loose	S2

medium dense	S3

dense	S4

very dense	S5

Rock

extremely low strength rock	R1

very low strength rock	R2

low strength rock	R3

medium strength rock	R4

high strength rock	R5

very high strength rock	R6

extremely high strength rock	R7

BED SPACING

massive / absent bedding	MA

very thickly bedded (>2m)	VB

thickly bedded (600-2000mm)	CB

medium bedded (200-600mm)	MB

thinly bedded (60-200mm)	TB

very thinly bedded (20-60mm)	UB

thickly laminated (6-20mm)	LM

thinly laminated (<6mm)	LL

irregular spaced bedding	IR

MOIST SENSITIVE

not sensitive	N

low sensitivity	L

medium sensitivity	M

high sensitivity	H

PLASTICITY

non plastic	N

brittle	B

low plasticity	L

intermediate plasticity 	I

high plasticity	H

DEFECT DATA

DEFECT TYPE

Natural

bedding plane	BP

broken zone	BZ

clay band	CL

coal cleat	CE

contraction fracture	CF

cross bedding	XB

dyke	DY

fault	FT

foliation	FO

fracture (undifferentiated)	FR

joint	JN

shear zone	SH

sill	SI

softened zone (non-tectonic)	SO

vein	VN

Induced and Non-Intact

discing	DS

drilling induced break	DB

drilling induced broken zone	DZ

DEFECT INTACT

intact	I

DEFECT CONTIN

continuous across core width	C

discontinuous across core width	D

divaricates (splits)	V

truncated within core width	T

DIP ORIENT METH

directly measured from reference line	D

estimated	E

indirectly measured	I

measured from televiewer	A

SHAPE

concave / convex	C

irregular	I

planar	P

stepped	S

undulose	U

ROUGHNESS

polished	P

rough	R

slickensided	K

smooth	S

JRC

1 … 10

INFILL TYPE

apatite	AP

calcite	CA

carbonaceous remains	XR

carbonate	CB

chlorite	CR

clay	CL

coal	CO

crushed rock	CU

dickite	DI

fossils 	FO

glauconite	GC

gypsum	GY

haematite	HE

illite	IL

iron oxide	IO

kaolinite	KA

limonite	LI

magnetite	MT

manganese	MG

marcasite	MC

mica	MI

montmorillonite	ML

other	OT

plant fragments	PF

pyrite	PY

quartz	QZ

sand	SA

siderite	SD

silt	SI

talc	TA

zeolite	ZE

INFILL MODE

absent	A

blebs	L

breccia	B

gouge	G

healed (cemented)	H

open	O

rubble	R

surface completely coated	C

surface partly coated	P

surface staining	S

trace	T

POINT LOAD DATA

SAMPLE STATE

dry	D

wet	W

PL TEST TYPE

axial	A

diametral	D

irregular	I

FAILURE MODE

bedding plane	B

invalid	I

joint	J

penetrative	P

valid	V

SAMPLE TYPE

Ply sample	PLY

Composite sample	COMP

SAMPLE_LITH

Coal	CO

Stone	SN

Parting	PA

Roof	RF

Floor	FL

PREV_TESTING

Gas desorption	GD

Geotech	GT

LAB_NAME

ALS – Bowen	ALSBWN

ALS – Collie	ALSCIE

ALS – Emerald	ALSEMD

ALS – Gladstone	ALSGLT

ALS – Gunnedah	ALSGUH

ALS – Lithgow	ALSLTW

ALS – Mackay	ALSMKY

ALS – Newcastle	ALSNTL

ALS – Richlands	ALSRC

ALS – Riverview	ALSRVW

Bureau Veritas – Blackwater	BVBLT

Bureau Veritas – Gladstone	BVGLT

Bureau Veritas – Muswellbrook	BVMBK

Bureau Veritas – Mackay	BVMKY

Bureau Veritas – Moranbah	BVMBH

Bureau Veritas – Newcastle	BVNTL

Bureau Veritas – Singleton	BVSGL

GeoGAS – Mackay	GGMKY

GeoGAS – Wollongong	GGWOL

Preplab Testing Services – Gladstone	PTSGLT

Preplab Testing Services – Rockhampton	PTSROK

SGS – Bowen	SGSBWN

SGS – Gladstone	SGSGLT

SGS – Muswellbrook	SGSMBK

SGS – Mackay	SGSMKY

SGS – Mount Thorley	SGSMTH

SGS – Newcastle	SGSNTL

SGS – Wollongong	SGSWOL

Unknown	UNK

NB : For results from CCI use the appropriate  Bureau Veritas code as CCI was taken over by Bureau Veritas

in 2007.

PRE_TREATMENT

Crushed		CRSH

Drop Shatter	DROP

Drop Shatter, Hand Knapped	DPHK

Drop Shatter, Hand Knapped, Dry Tumbled	DPHD

Drop Shatter, Hand Knapped, Wet Tumbled	DPHW

Coal Pulverisation sieve analysis	CLPV

Coke Properties sieve analysis	CKPP

Gas - Uncrushed sample	GSUN

Gas - Crushed sample	GSCR

SIZE_TYPE

Wet	W

Dry	D

GRAY_KING_TYPE

A	A

B	B

C	C

D	D

E	E

F	F

G	G

G1	G1

G2	G2

G3	G3

G4	G4

G5	G5

G6	G6

G7	G7

G8	G8

G9	G9

G10	G10

G11	G11

G12	G12

G13	G13

G14	G14

G15	G15

G16	G16

G17	G17

G18	G18

G19	G19

G20	G20

VIT_TYPE

Vitrinite Reflectance A (Telovitrinite)	TELO

Vitrinite Reflectance B (Detrovitrinite)	DETRO

Vitrinite Reflectance (A & B)	ALL